SOP-CP-001: Case Picker | Target Partial Pallet Order Fulfillment | Cadeco Industries | INDE 4334 Team 7

STANDARD OPERATING PROCEDURE

SOP-CP-001: Case Picker

Target Partial Pallet — Case Picking and Assembly

PROPOSED STATE | Post-Implementation

| Document ID | SOP-CP-001 | Revision | Rev. 1.0 |
| --- | --- | --- | --- |
| Effective Date | April 2026 | Status | PROPOSED |
| Facility | Cadeco Industries — Deer Park, TX | Prepared By | INDE 4334 Capstone Team 7 |

# 1. Purpose

This SOP defines the case picking procedure for personnel assigned to Target partial pallet order fulfillment at the Deer Park facility. The Case Picker is responsible for receiving the picklist from the Team Lead, picking the correct cases from the designated rack cell, assembling them onto a pallet at the build station, and handing off the completed pallet for QC inspection and wrapping.

This role requires close coordination with the Team Lead and the Forklift Driver throughout the order cycle.

In Scope

All Case Pickers assigned to the Target zone at Deer Park. All partial pallet orders for the 7 Target SKUs: 62632, 62641, 62004, 62106, 62432, 62231, 62232.

Out of Scope

Full-case or full-pallet picks that do not involve partial pallets. Non-Target zone picking operations.

# 2. Roles and Responsibilities

| Role | Responsibility in This Context |
| --- | --- |
| Case Picker | Receive picklist from Team Lead. Locate correct SKU rack cell. Verify FIFO sequence. Count and pick required cases. Assemble partial pallet. Stage for QC. Report FIFO exceptions immediately. |
| Team Lead | Issue picklist and labels. Monitor progress. Conduct QC inspection. Authorize wrapping. See SOP-TL-001. |
| Forklift Driver | Retrieve pallets from designated rack cells when signaled. Return depleted pallets. Operate stretch wrap machine. See SOP-FD-001. |

# 3. Required Tools and Equipment

| Item | Purpose | Notes |
| --- | --- | --- |
| Picklist (from Team Lead) | Identifies SKU, quantity, and rack cell address for each order | Never begin picking without a signed and approved picklist. |
| Empty pallet (from staging) | Platform for assembled cases | Inspect for damage before use. Reject cracked or spliced pallets. |
| Pallet jack | Moving assembled pallet to staging zone | PROPOSED: Dedicated pallet jack stored at new staging area. |
| Radio / communication device | Communicate with Team Lead and Forklift Driver | Must be charged and on-hand before shift start. |
| Rack location reference card | Quick reference for designated cell addresses | PROPOSED: Posted at Team Lead workstation and build station. Lists all 7 SKU cell addresses (R30-C01-A through R31-C03-C). |

# 4. SKU Designated Cell Reference

Under the proposed layout, all 7 Target SKUs are assigned to fixed cells in Rack 30 and Rack 31, adjacent to the staging area. The Case Picker should use the rack reference card at the build station to confirm the cell address before traveling to the rack.

| SKU | Product | Class | Cell Address | Rack and Level |
| --- | --- | --- | --- | --- |
| 62632 | Org Sugar, 6ct-2lb | A | C01-A | Rack 30, Level A (ground) — nearest build station |
| 62641 | Lt Brown Sugar, 6ct-1.5lb | B | C03-B | Rack 30, Level B |
| 62004 | Powdered Sugar, 6ct-1lb | B | C05-B | Rack 30, Level B |
| 62106 | Coconut Palm Sugar, 6ct-1lb | C | C07-C | Rack 30, Level C |
| 62231 | Light Agave, 6ct-23.5oz | C | C01-C | Rack 31, Level C |
| 62432 | Org Sugar, 6ct-4lb | C | C02-C | Rack 31, Level C |
| 62232 | Raw Agave, 6ct-23.5oz | C | C03-C | Rack 31, Level C |

# 5. FIFO Compliance Protocol

FIFO (First-In, First-Out) requires that the oldest product by manufacture or receive date is always picked first. This is a mandatory SQF compliance requirement. Failure to follow FIFO is the leading cause of NVA rework time in the current process, observed at 14.22 minutes per FIFO failure event.

## 5.1 How to Identify FIFO Order

- Check the manufacture or receive date on the case or pallet slip. The earliest date is the oldest stock and must be picked first.

- If dates are not visible, the pallet in the deepest rack position is typically the oldest. Verify with the Forklift Driver if uncertain.

- If two pallets share the same date, pick from left to right.

## 5.2 FIFO Exception Procedure

If a pallet in FIFO order is damaged, on QA Hold, or physically inaccessible without a forklift move:

- Do not skip to the next pallet without authorization.

- Radio the Team Lead immediately. Report the SKU, rack cell address, and the nature of the FIFO exception.

- The Team Lead will coordinate with the Forklift Driver to resolve the access issue or authorize the exception.

- Document the exception on the picklist: write FIFO EXCEPTION followed by the reason and initial the entry.

# 6. Picking Procedure

Times below are observed averages from WorkStudy+ 7 (January through February 2026). Proposed changes are noted in the Notes column.

| Step | Activity | Class. | Obs. Time | Notes |
| --- | --- | --- | --- | --- |
| 1 | Receive picklist and labels from Team Lead. Verify SKU, quantity, and order number match. | VA | 0.5 min | Never begin picking without a verified picklist. |
| 2 | Locate empty staging pallet. Inspect for structural integrity: no broken boards, no sharp edges. | NEC | 0.5 min | No proposed change. |
| 3 | Travel to designated rack cell per picklist. | NEC | 1.8 min | CURRENT: Racks dispersed, average 45 to 85 ft per SKU. PROPOSED: Rack 30 and Rack 31 adjacent to build station, average approximately 15 ft. |
| 4 | Confirm rack cell address matches the picklist SKU. Verify product code on case matches label. | VA | 0.5 min | Critical allergen control step. No proposed change. |
| 5 | Confirm FIFO order per Section 5.1. Identify the oldest pallet. | NEC | 0.75 min | PROPOSED: Adjacent rack layout reduces scanning time across rack positions. |
| 6 | Signal Forklift Driver to retrieve and position pallet. Wait for pallet to be accessible at floor level. | NEC | 2.27 min | Observed wait time, WorkStudy+ 7. PROPOSED: Reduced with adjacent rack layout. |
| 7 | Count and pick the required number of cases from pallet. Place on staging pallet in stable stacking pattern. | VA | 3.5 min | Core value-added step. Time depends on case count per order. |
| 8 | If FIFO exception detected, execute Section 5.2 protocol. If pallet is depleted, mark as depleted and signal Forklift Driver. | NEC | 0 min if no exception | CURRENT: Average 14.22 min NVA when a FIFO failure occurs. PROPOSED: Reduced frequency with designated cell layout. |
| 9 | Apply SKU label to assembled partial pallet. Confirm label matches picklist. | VA | 0.5 min | Required for traceability. No proposed change. |
| 10 | Move completed partial pallet to staging area using pallet jack. Position for Team Lead QC inspection. | NEC | 1.0 min | PROPOSED: Designated floor-taped staging zone adjacent to wrap machine reduces travel. |
| 11 | Notify Team Lead via radio that pallet is ready for QC inspection. | NEC | 0.25 min | No proposed change. |

# 7. Quality Checkpoints

The Case Picker is responsible for the following quality gates during every order.

| No. | Checkpoint | Action if Failed |
| --- | --- | --- |
| 1 | SKU code on the picked case matches the picklist SKU. | Stop picking. Radio Team Lead. Do not mix SKUs on a pallet. |
| 2 | Case count matches the picklist quantity. | Recount. If discrepancy persists, report to Team Lead before staging. |
| 3 | Cases are undamaged: no tears, moisture damage, or crushed corners. | Segregate damaged cases. Pick only undamaged product. Note on picklist. |
| 4 | FIFO sequence has been verified. | Follow the FIFO exception protocol in Section 5.2. |
| 5 | Pallet label matches the order number and SKU. | Do not move pallet to staging until label is correct. Request replacement label from Team Lead. |

# 8. Performance KPIs

Baselines drawn from WorkStudy+ 7 observed data. Targets to be set by Cadeco management following implementation.

| KPI | Current Baseline | Measurement Method |
| --- | --- | --- |
| Pick travel time per batch | 1.8 min average (observed) | WorkStudy+ 7 observation |
| FIFO exception rate | Approximately 2.4 events per week (observed) | Incident log — to be maintained by Case Picker |
| Case pick accuracy | Not formally tracked at baseline | QC rejection log — to be established |
| Case Picker cycle time per batch | Approximately 10.82 min (sum of observed steps) | Time study per batch |

# 9. Safety and Compliance

OSHA 29 CFR 1910.178 — Forklift exclusion zone: Always maintain clearance from the forklift during pallet retrieval. Do not enter the forklift aisle while the Forklift Driver is operating.

SQF Code Edition 9, Section 2.6 — Allergen control: Case Picker is responsible for SKU label verification at the point of pick. Do not mix SKUs on a single pallet.

SQF Code Edition 9, Section 2.4.3 — Ergonomics: Cases weigh 10 to 30 lbs depending on SKU. Use proper lift technique: bend at the knees, keep the load close to the body, do not twist while bearing weight. Use team lifts for cases exceeding 35 lbs. Report any musculoskeletal discomfort to the Warehouse Supervisor.

Maximum stack height: Do not stack cases higher than 60 inches on the staging pallet without supervisor approval.

New wrap machine area: Do not cross into the wrap machine safety exclusion zone during machine operation. See floor tape markings.

# 10. Document Control

| Rev. | Date | Author | Change Summary |
| --- | --- | --- | --- |
| 1.0 | April 2026 | INDE 4334 Team 7 | Initial Proposal |

Related Documents: SOP-TL-001 | SOP-FD-001 | CL-TARGET-001 (Documentation Guide) | Staging_Cell_Layout.drawio

Cadeco Industries | INDE 4334 Team 7 | SOP-CP-001 Rev. 1.0 | Page 1